# ASSIGNMENT 1-REPORT

ANSWER 1) DOGS

ANSWER 2) USART VS UART USART has both synchronous and asynchronous modes, unlike UART which is purely asynchronous. So USART can do what UART does, and more. Thus, speed is more in case of USART but UART is much simpler to implement, understand and handle and is cheaper to use too. For high speed data transfer, USART is used , else, UART should be used. For the rover , I would prefer using USART since it offers more flexibility and higher speed for data transfer.

HOW IS THE CPU OPERATION HANDLED WHILE TRANSMITTING/RECEIVING DATA?

The different techniques implemented in code is divided into two main categories – blocking and non blocking.

Polling is a blocking technique where the cpu’s work is stalled while it transmits or receives data in uart. The timeout parameter is the amount of time the cpu Is told to wait until resuming its normal work i.e. the amt of time we have estimated for the transmission of data to take place. Lets say value of timeout parameter is 1000ms. If data doesn’t get transferred in that time, then value of the transmit/receive function is HAL_TIMEOUT() and if it does get transferred successfully within the time specified, the CPU directly shifts to the next line of code (i.e. it doesn’t wait for 1 second to get completed) and the value is HAL_OK(). We will be using these two values in our code logic.

------------------------------------------------------------------------------------------------------------

EXTRA INFORMATION RELATED TO CODE :

&huart1 for USART1

Timeout is number of milliseconds the cpu should wait until resuming its normal work.

uint8_t is used to store numbers. Note that it displays the ascii character rather than the number since uart transfers the data in ascii format.                                                          Example :- 	uint8_t num = 65;                                                                                                                                                                                                        XXXXXXXXXXXHAL_UART_Transmit(&huart1,    num,    1,    10);

will display A and not the number 65.                                                                                                       To store strings, use uint8_t arrays. In the array, each individual character is stored as its ascii value which is then used later while displaying the string . In code, we can convert number we want to print to its ascii code and use that in our transmit function using:-

uint8_t num = 65;   uint8_t numarray[4];

sprintf( numarray       , “%d\n” , num);

and then use numarray in transmit function.

NOTE :  u need to – #include <stdio.h>    (after include <main.h>)       for sprintf function to work

--------------------------------------------------------------------------------------------------------------

ANSWER 3) Any data which comes externally, is stored in a hardware buffer. In blocking techniques, the cpu waits until the required amount of bits reach and get stored in the buffer after which it then takes them out of the buffer. If timeout is set as zero, its no longer blocking since the time it has to wait is zero. On calling the function now, it will take whatever value is present inside the buffer. If there is a value present, it grabs it and value returned is HAL_OK and if there is no value or if the number of bits present is less than the number declared in the function, it returns HAL_TIMEOUT.

| ANSWER 4) we use HAL_UART_RECEIVE to receive data. Timeout is set as 5 seconds initially. If no data/incomplete data is received then value of function is HAL_TIMEOUT and if value is received then value is HAL_OK. We check these values and use if-else loop to turn led on/off ,or to transmit a message , according to the value of the function. For HAL_OK , we simply perform the instructions given and executed the while loop again. We received data within 5 seconds and so there is no issue. |
| --- |
| For HAL_TIMEOUT, it means that it has been 5 seconds since the last data got recieved so we transmit a lonely message and then create a while loop within which we keep on checking for data every 2 seconds using the receive function. (Here, we set timeout to 2 seconds). And the core logic remains same. Till the value of the function isn’t HAL_OK, we remain in our while loop and keep transmitting a lonely message and checking for data every 2 seconds. As soon as we get data , the value is HAL_OK, we go out of the while loop and then perform the instructions – led on/off or invalid message according to the value received from the user. Thus , we use the timeout parameter itself to wait for2 seconds/ 5 seconds at respective parts of the code rather than using HAL_GET_TICK() function. |  |  |

While blocking techniques can be used for transmission of small amts of data, sending large amts will delay the normal work of the cpu since the cpu can only execute the next line of code once it completes uart transmission. If we want both work to be done simultaneously, we will have to use interrupts or DMA. Moreover, some data bits can be lost if the data is not sent at the moment the cpu is at the uart receive function ,if data is sent when the cpu is at some other line of code or if the timeout parameter set wasn’t enough for entire data to get transferred, resulting in no data being read at all at the receivers end. One fix is using HAL_MAX_DELAY which basically makes the cpu wait forever till it receives the expected amt of data. It freezes the cpu’s work and isn’t a good fix usually.

Interrupts –[  We know that while transmitting data, the data is first sent to the transmit register(buffer), which automatically shifts it to the shift register which then transfers it serially via UART wire to the receiver . There, is another shift register which takes the serial data and outputs it in parallel to the receive register and the cpu takes data from this buffer. ]  Whenever, we want to transmit or receive data ,we call HAL_UART_TRANSMIT_IT or HAL_UART_RECEIVE_IT functions in the code. This tells the uart hardware to signal an interrupt to the cpu whenever there is space for the cpu to  transmit/receive data. When an interrupt is sent/received, what happens in code? There exists a piece of code which is designed to handle interrupts-interrupt handler. Putting data into the buffer or taking data out of the buffer is done by the handler. The CPU moves to the interrupt handler and after it executes whatever it had to, the cpu goes back to its main line of code. Thus, by making use of interrupts, the cpu’s work now only gets blocked while executing the interrupt handler code, and not during transmission of data as in the case of blocking techniques.

HAL_UART interrupt functions (transmit and receive) don’t have timeout parameter. And its logical to see why since they just tell the uart hardware to be on a lookout for data and specifications of the data is given as parameters in the function itself. They don’t make cpu wait and so don’t need a timeout parameter.                                                      EXAMPLE:- HAL_UART_Receive_IT(&huart1, data, 5). Lets say we wanna transmit the word hello (5 bits) from laptop to STM and store it in data variable. U could take as much time you want to send the first four bits and could send them separately too. Only when the last bit- the letter O is sent, is the interrupt signaled.

ANSWER 5) FOR TRANSMIT : - The HAL_UART_TRANSMIT_IT function essentially transfers data from cpu to the transmit register(buffer). As soon as the data is transferred from the cpu to the buffer, the cpu resumes its normal work and tells the uart hardware to send an interrupt when its done transmiting the data outside, so as to signal the cpu that its time to send the next piece of data. When the interrupt is sent, the cpu moves to the interrupt handler which executes some code, calls the transmit call back function. So,in the code, we use HAL_UART_TRANSMIT_IT function in if loop inside while loop and the condition of if loop Is met when callback function gets called(i.e. we use a variable and change its value when callback is called and check the value in if loop). Thus, the cycle repeats. FOR RECEIVE, we call the HAL_UART_Receive_IT function. This basically sets the UART hardware up and tells it to send an interrupt when the receive data register has the amount of bits we want to receive. When interrupt is signaled, the interrupt handler takes data from the buffer and sends it to RAM. However, after this is done, the function gets disabled i.e. the uart hardware can no longer be on the lookout for incoming data again. So we need to call the function again to enable the uart functioning. We do this by calling the HAL_UART_Receive_IT inside a callbackfunction which automatically gets called when the interrupt is signaled. Thus, the uart hardware always remains enabled this way.

ANSWER 6) We call receive IT function to set up UART hardware for receiving input. We put the logic(if-else) inside our call back function so that whenever an interrupt is sent, the data will be received and call back function will be called. In the if else statements , we check the value of the input and toggle LED or transmit a “INVALID” message accordingly. Receive IT function should be called again in the call back function. To keep track of time , we use HAL_GetTick() and update values of our variable -status in the call back function. In the while loop, we execute the if else statement to check when the last data was received and print “LONELY “accordingly. Value of status>= 5000 and status_lonely>=2000 for “LONELY” to be sent using Transmit IT function. The if else statement will be executed continuously since its in the while loop and we reset value of status_lonely to HAL_GetTick() in the if else block. LONELY will be transmitted continuously every 2 seconds until we receive an input from the user, in which case the receive call back function will be called and values of status and status_lonely will be reset.

DMA –

ANSWER7) Works similar to interrupts. Only here, there is a separate hardware which can take values from buffer and upload it into RAM and vice versa. So no CPU intervention is needed

ANSWER 8) The code essentially has the same logic we applied in interrupts and the structure is very similar too. One difference is in the called functions where HAL_UART_TRANSMIT_IT() gets replaced by HAL_UART_TRANSMIT_DMA() and HAL_UART_RECEIVE_IT() gets replaced by HAL_UART_RECEIVE_DMA(). And also, in cubemx, Tx and Rx had to be enabled in USART1 DMA settings.

ANSWER 9) Serial.read() is non blocking as when it is called, the cpu goes to the receive buffer and takes in whatever value is present. If its empty, it returns -1. You can use serial.available(). This looks at the buffer’s counter and returns the number of bytes that are available in the buffer. [Note that serial.read() only reads one byte at a time]. Serial.print is non blocking for small texts but it will block the cpu if its transmitting too many bytes and the transmit buffer is already full. So technically , yes serial.print is a blocking function but since for small texts, when there is enough space in our 64 byte transmit buffer, it doesnt feel like blocking since it puts the value in the buffer instantaneously. In Arduino, whenever hardware receive a value, an interrupt is fired and the data is copied onto a buffer. You can access that value using serial.read(). So interrupts are inbuilt and are default for the pins. This interrupt hardware is activated by serial.begin( ---). This is similar to STM cube IDE uSART where we setup uart hardare for interrupts using the HAL_UART_Transmit_IT() and HAL_UART_Receive_IT() functions.

ANSWER 10) The internal clock is configured at 72 MHz. We use counters to keep track of time and a typical 16bit counter will reach its max value in a millisecond. To use it to keep track of time in seconds, We use prescalar to reduce the frequency of the clock. The new frequency is 72 / (PSC+1) value in MHz. Prescalar is a 16bit register an the psc value is what u load into the register.The timer runs at this new frequency. It ticks one time every (PSC+1) cycles. Thus, it controls the time for each tick. However we still don’t have a way to actually measure the number of ticks i.e. measure time. Moreover, prescalar value is limited to powers of 2 so we cant achieve every possible frequency alone. Auto reload register sets the maximum value for the timer to count till. When the timer reaches ARR, it resest back to zero and starts again. Alue of counter updates by 1. Thus, its used to set the width of PWM cycle. CCRx is used to set the duty cycle. DUTY CYCLE = CCRX / ARR. After the counter hits value = CCrx value, the signal goes form high to low or vice versa. EXAMPLE to implement 30 percent duty cycle with 1000ARR, we set CCRx to 300 . Till the counter hits 300, the signal remains high and after that low. PWM FREQUENCY = f clock/ ((1+Psc)(1+ARR)) DUTY CYCLE PERCENTAGE = (CCRx/ARR) *100 NOTE that even tho both presecalar and Auto reload register technically reduce the frequency, we still have to use both since the max value of the register can be a 16bit value and value of one without the other would have to be very high which is not feasible. ANSWER 11) HAL_TIM_PWM_Start(&htim1, TIM_CHANNEL_1); this sets pwm waveform on pin PA8. In cube mx, we set psc value to 71, arr to 999 and CCRx i.e. pulse value to 300.